Design and Simulation of High-Efficiency Buck Converter (DC-DC Step-Down)

OBJECTIVE

To design a high-efficiency non-isolated buck converter to step down 12V to ~6V, focusing on switching performance, ripple minimization, and loss optimization.

SYSTEM OVERVIEW

- The system architecture includes:

- A PWM-controlled MOSFET (AO6408) as a switching device

- A Schottky diode (MBR745) to handle freewheeling current

- An LC filter to minimize ripple at the output (143 µH inductance, 200 µF capacitance)

- The system is operating in Continuous Conduction Mode (CCM) to minimize ripple in the current delivered to the load.

KEY EQUATIONS

- Voltage Conversion:

Vout   = D.Vin

- Inductor Ripple Current:

IL =   (Vin – Vout).D

L.fs

- Output Voltage Ripple:

Vout  ≈           IL

8 fs.C

DESIGN SPECIFICATIONS

- Vin: 12 V

- Vout: ~5.7 V

- Load: 1 Ω (~5.7 A)

- Switching Frequency: 25 kHz

- Duty Cycle: ~0.5

- Output Power: ~32-34 W

SCHEMATIC

PERFORMANCE

- Efficiency: ~93-94%

- Total Losses: ~2-2.5 W

- Dominant: Diode conduction loss (~1.5 W)

- Output Ripple: <2%

- Transient Response: <1 ms settling time

OUTPUT

ERROR ANALYSIS

- Non-ideal component losses: MOSFET switching losses, diode forward drop, and inductor DCR reduce efficiency and cause deviation from ideal output.

- Open-loop operation & parasitics: Fixed duty cycle and effects like capacitor ESR and noise lead to output voltage variation and ripple under load/input changes.

IMPROVEMENTS

- Closed-loop feedback control (PWM regulation): This ensures a stable output voltage under varying line and load conditions, as well as a faster transient response.

- Employ synchronous rectifier (substitute MOSFET for diode switch): This minimizes conduction losses, resulting in a substantial increase in efficiency.

CONCLUSION

The project has shown a practically efficient design of a buck converter circuit and has emphasized some important concepts in power electronics that are often encountered in industry and real-world applications.